CHARACTERISTICS AND APPLICATIONS:

VEGECLAR C

Fining agent allergen free

VEGECLAR C is a new generation clarifier that combines the positive characteristics of yeast protein extracts with the clarifying and stabilizing capacity of Chitosan derived from Aspergillus niger.

VEGECLAR C is a valid alternative to animal origin gelatines used in flotation or in the static clarification of juices or wines, natural and free from allergens (Annex II Reg 1169/2011), it can be used in the production of organic wines (Reg. EU 1584 / 2018).

VEGECLAR C is able to rapidly flocculate, thanks to its reactivity towards suspended particles, reducing oxidized and oxidizable polyphenolic fractions, preserving the aromatic potential of white and rosé wines.

USE AND DOSES:

- 5-15 g/hl in juices;

- 5-15 g/hl in wines.

Dissolve VEGECLAR C in water under stirring, at room temperature or preferably between 20 and 30 ° C, in a ratio of 1:10 to 1:20. Add slowly the solution obtained to the mass, keeping it in movement.

COMPOSITION:

REV. 01_050619

Protein extracts of yeast, chitosan, selected silicas, organic acids (citric acid, tartaric acid).

PAKAGING:

Bag 10 kg, Pack 1 kg.

This is a product that is not considered dangerous so it does not need a safety data sheet.